Drive Setup & Samba Share

Fedora Linux — Full Setup Notes

April 6, 2026

# 1. Drive Diagnosis

Before setting up anything, the drives were inspected to understand their state.

Check what drives are connected

sudo lsblk

Result showed three block devices:

| Drive | Status |
| --- | --- |
| sda | Reported 0B — appeared dead or disconnected |
| sdb | 465.8G — system drive (boot, home, root) |
| sdc | 465.8G — new empty drive, ready to use |

Investigate sda with dmesg

sudo dmesg ¦ grep -i sda sudo fdisk -l /dev/sda sudo smartctl -a /dev/sda

⚠️  sda showed DID_BAD_TARGET I/O errors on sectors 0, 2, 4, 6 immediately at boot. It initially reported 4.14 GB then collapsed to 0B. fdisk and smartctl both failed to open it.

⚠️  Verdict: sda is a dead or DOA (dead on arrival) drive. Physical inspection or replacement needed.

# 2. Partition and Format sdc

Since sdc is a brand new drive, it had no partition table or filesystem. It needed to be partitioned and formatted before it could be mounted.

Step 1 — Create a GPT partition table and partition

sudo fdisk /dev/sdc # Inside fdisk interactive prompt: g → create new GPT partition table n → new partition 1 → partition number 1 [Enter] → accept default first sector (2048) [Enter] → accept default last sector (use full disk) w → write changes and exit

Step 2 — Format the partition as ext4

sudo mkfs.ext4 /dev/sdc1

ℹ️  ext4 is the standard Linux filesystem. This created a 458G filesystem with UUID: 7c425370-9162-4f17-b131-ca096c06e308

Step 3 — Create a mount point and mount the drive

sudo mkdir /mnt/drive1sdc sudo mount /dev/sdc1 /mnt/drive1sdc

Step 4 — Verify the mount

df -h ¦ grep sdc

✅  Output confirmed: /dev/sdc1  458G  2.1M  435G  1%  /mnt/drive1sdc — drive is mounted and ready.

Get the drive UUID (for fstab later)

sudo blkid /dev/sdc1

ℹ️  UUID=7c425370-9162-4f17-b131-ca096c06e308  TYPE=ext4. Keep this for permanent mounting via /etc/fstab.

# 3. Install and Configure Samba

Samba allows Windows machines to access Linux drives over the network using standard Windows file sharing (SMB protocol).

Install Samba

sudo dnf install samba sudo systemctl enable smb sudo systemctl start smb

Edit the Samba config file

sudo nano /etc/samba/smb.conf

Add the following block at the bottom of smb.conf:

[drive1sdc] path = /mnt/drive1sdc browseable = yes writable = yes guest ok = yes public = yes force user = nobody

Set permissions on the mount point

sudo chmod 777 /mnt/drive1sdc

Open firewall for Samba

sudo firewall-cmd --permanent --add-service=samba sudo firewall-cmd --reload

Create a Samba user account

Even with guest ok, Windows often prompts for credentials. Create a Samba password for your Linux user:

sudo smbpasswd -a neo

ℹ️  On Windows, log in with username: neo and the password you set here.

Start NMB (NetBIOS — helps Windows discover the share)

sudo systemctl enable nmb sudo systemctl start nmb

Restart Samba to apply all changes

sudo systemctl restart smb nmb

# 4. Fix SELinux Blocking Samba

Fedora uses SELinux which blocks Samba from accessing directories by default, even after chmod 777. Two fixes were needed.

Fix 1 — Allow Samba read/write via SELinux boolean

sudo setsebool -P samba_export_all_rw 1

Fix 2 — Apply correct SELinux file context to the mount point

sudo dnf install policycoreutils-python-utils # if semanage not found sudo semanage fcontext -a -t samba_share_t "/mnt/drive1sdc(/.*)?" sudo restorecon -R /mnt/drive1sdc sudo systemctl restart smb

ℹ️  The samba_share_t context tells SELinux that this directory is intentionally shared via Samba. Without this, smbd logs show: 'does not exist or permission denied' even though the path exists.

# 5. Access the Share from Windows

Once Samba is running and SELinux is configured, access the share from any Windows PC on the same network.

Find the Fedora machine's IP address

ip a

ℹ️  Look for the inet address on your network interface (e.g. eth0 or enp3s0). Example: 10.0.0.122

Connect from Windows

Open File Explorer on Windows and type in the address bar:

\\10.0.0.122

The drive1sdc share will appear. Enter credentials if prompted (username: neo, Samba password set earlier).

✅  Drive successfully shared and accessible from Windows.

# 6. Optional — Make Mount Permanent

By default, the mount is lost on reboot. To make it permanent, add it to /etc/fstab:

# Add this line to /etc/fstab: UUID=7c425370-9162-4f17-b131-ca096c06e308 /mnt/drive1sdc ext4 defaults 0 2

ℹ️  Replace the UUID with the one from your blkid output if different. The '0 2' at the end means: don't dump, fsck on boot after root.

# Quick Reference — All Commands

# --- DIAGNOSIS --- lsblk sudo dmesg ¦ grep -i sda sudo fdisk -l /dev/sda sudo smartctl -a /dev/sda # --- PARTITION & FORMAT sdc --- sudo fdisk /dev/sdc # g, n, [Enter], [Enter], w sudo mkfs.ext4 /dev/sdc1 sudo mkdir /mnt/drive1sdc sudo mount /dev/sdc1 /mnt/drive1sdc df -h ¦ grep sdc sudo blkid /dev/sdc1 # --- SAMBA --- sudo dnf install samba sudo systemctl enable smb nmb && sudo systemctl start smb nmb sudo nano /etc/samba/smb.conf sudo chmod 777 /mnt/drive1sdc sudo firewall-cmd --permanent --add-service=samba sudo firewall-cmd --reload sudo smbpasswd -a neo sudo systemctl restart smb nmb # --- SELINUX --- sudo setsebool -P samba_export_all_rw 1 sudo dnf install policycoreutils-python-utils sudo semanage fcontext -a -t samba_share_t "/mnt/drive1sdc(/.*)?" sudo restorecon -R /mnt/drive1sdc sudo systemctl restart smb # --- WINDOWS ACCESS --- ip a # find your IP # Then in Windows File Explorer: \\<your-ip>